Defect Measurement — Simplified Approach

PIP geometry layer · replaces Geometry_final’s 3D reconstruction path

1. The decision

Defects are measured, not reconstructed. Almost every residential defect reduces to a few numbers on a known surface: area, length, width, angle, offset, depth. No point cloud, mesh, COLMAP, DA3 or multi-view fusion in the per-defect path. The house-level 3D model is separate: it only places each defect on the right wall or plane; measurements stay 2D on that plane.

2. Pipeline (one photo per defect is enough)

- Box: hand-drawn for now (existing prompts/<id>.json); replaced by the YOLO/GDINO perception pipeline later. Same input format, so the swap is one line.

- Mask: SAM2 from the box (+ positive/negative clicks). A box alone overstates area on diagonal cracks and irregular blobs.

- Scale + rectify: detect the card on the defect’s surface → homography to a straight-on view → fixed cm/px. Fallbacks: LiDAR distance + phone tilt, then ruler, then known building dimensions.

- Measure (table below) → output JSON with each number’s scale source and confidence.

| Measurement | Applies to | How |
| --- | --- | --- |
| Area | Peeling paint, stains, mold, moss, missing shingles, broken-away drywall/stucco, rot on a face | Mask pixels × (cm/px)² after rectification |
| Length / width | Cracks, gaps, long defects | Length along mask skeleton (centerline); width = min/max thickness of mask |
| Angle | Leaning post, railing, chimney; floor/gutter slope | Member direction vs gravity from phone accelerometer (in camera frame). Lean = deviation from vertical |
| Offset / bulge | Bowed wall, sagging ridge/deck, step across foundation crack | 3 points (ends + middle) or a LiDAR line; distance of middle from straight line |
| Depth (max) | Holes, rot | Worker probe band: <1 cm / 1–3 cm / >3 cm / through. Never from image depth |

3. Where the effort goes: getting the edges right

Measurement accuracy is now mostly mask accuracy. SAM2 is good on crisp edges and weak on thin or diffuse ones, so treat three edge types differently:

- Crisp (broken-away drywall, missing shingle, hole opening): SAM2 as is.

- Thin (cracks, gaps): SAM2 tends to miss or fatten them. Inside the box, use a ridge/line filter (e.g. Frangi) + threshold + skeleton for length, measure width on the close-up.

- Diffuse (stains, moss, mold, paint fade): no true edge exists. Threshold color/texture difference from the clean surface around the box; report area as a range (tight vs loose threshold).

Segment on the original photo, then warp the mask with the homography; do not segment the warped image.

4. Rules

- Flatness: assume flat by surface type (walls, ceilings, roof planes, slabs). If LiDAR is available, out-of-plane deviation <10% of defect size → flat; 10–25% → flat, flagged approximate; >25% → size band only.

- Angles must use the gravity vector rotated into the camera frame of that photo. The current code mixes sensor-world and DA3-world frames, which is why walls read ~90°.

- One scale per photo from the card; reject detections that imply implausible ruler length (current colour-blob detector finds false rulers).

- Unmeasurable cases (curved members, corners, unprobed cavities): size band + photo, marked ‘not measured’.

5. What stays and what goes

Keep: SAM2, card/ruler scale logic (with plausibility check), capture metadata, severity thresholds (recalibrate). Drop: Qwen in this stage, DA3, COLMAP, fusion, plane fitting on point clouds, meshing.

Done when: the 8 test defects reproduce the pilot’s hand estimates (ceiling hole ≈5–7 cm, roof crack ≈25–28 cm) within ~15%, with every number carrying its scale source.